Practical 1: Create Different Blinking Patterns with LEDs by Connecting to GPIO Pins of Raspberry Pi

Aim

To interface 8 LEDs with current-limiting resistors to the GPIO pins of Raspberry Pi 4 Model B and implement different LED blinking patterns using Python. The patterns are displayed in a loop with a 3-second interval using RPi.GPIO in BCM mode.

Hardware Requirements

- Raspberry Pi 4 Model B

- 8 LEDs (Red / Yellow / Green)

- 8 × 220 Ω Current Limiting Resistors

- Breadboard

- Male-to-Female Jumper Wires

- 5V 3A USB-C Power Supply

Software Requirements

- Raspberry Pi OS (Debian-based)

- Python 3

- RPi.GPIO Library

- Thonny Python IDE or Terminal

Physical Pin Connection Table

| LED Number | GPIO Pin (BCM) | Physical Pin |
| --- | --- | --- |
| LED 1 | GPIO 17 | 11 |
| LED 2 | GPIO 27 | 13 |
| LED 3 | GPIO 22 | 15 |
| LED 4 | GPIO 10 | 19 |
| LED 5 | GPIO 9 | 21 |
| LED 6 | GPIO 11 | 23 |
| LED 7 | GPIO 5 | 29 |
| LED 8 | GPIO 6 | 31 |
| Common Ground | GND | 6 |

Circuit Diagram and Working Principle

- The long leg (Anode) of each LED is connected to its respective GPIO pin through a 220 Ω resistor.

- The resistor limits the current flowing through the LED and protects it from damage.

- The short leg (Cathode) of all 8 LEDs is connected to a common Ground (GND) rail on the breadboard.

- The common Ground rail is connected to Physical Pin 6 of the Raspberry Pi.

LED Patterns

The program displays the following five patterns:

| Pattern | LED States |
| --- | --- |
| All LEDs ON | 11111111 |
| Alternate LEDs ON | 10101010 |
| Opposite Alternate LEDs ON | 01010101 |
| First Four LEDs ON | 11110000 |
| All LEDs OFF | 00000000 |

import RPi.GPIO as GPIO

import time

GPIO.setmode(GPIO.BCM)

GPIO.setwarnings(False)

led_pins = [17, 27, 22, 10, 9, 11, 5, 6]

for pin in led_pins:

GPIO.setup(pin, GPIO.OUT)

def ledpattern(states):

for i in range(8):

GPIO.output(led_pins[i], states[i])

try:

while True:

patterns = [

[1,1,1,1,1,1,1,1],

[1,0,1,0,1,0,1,0],

[0,1,0,1,0,1,0,1],

[1,1,1,1,0,0,0,0],

[0,0,0,0,0,0,0,0]

]

for p in patterns:

ledpattern(p)

time.sleep(3)

except KeyboardInterrupt:

print("Program stopped")

finally:

GPIO.cleanup()

Practical 2 Use Telegram to control Raspberry Pi. GPIO Pins.

Aim

To set up a Telegram Bot using BotFather, interface one LED with a current-limiting resistor to a Raspberry Pi 4 Model B GPIO pin, and execute Python code using the telepot library to control the LED through Telegram chat commands (/on and /off).

Hardware Requirements

- Raspberry Pi 4 Model B

- 1 × LED (Red / Green / Yellow)

- 1 × 220 Ω Resistor

- Breadboard

- Male-to-Female Jumper Wires

- 5V 3A Power Adapter

- Active Internet Connection (Wi-Fi or Ethernet)

Software Requirements

- Raspberry Pi OS

- Python 3

- telepot Python Library (pip install telepot)

- RPi.GPIO Python Library

- Telegram App (Mobile or Desktop)

Physical Pin Connection Table

| LED Number | GPIO Pin (BCM) | Physical Pin |
| --- | --- | --- |
| LED 1 | GPIO 17 | 11 |
| Ground | GND | 6 |

Circuit Diagram and Working Principle

- The long leg (Anode) of the LED is connected to GPIO 17 through a 220 Ω resistor.

- The resistor limits the current and protects the LED from damage.

- The short leg (Cathode) of the LED is connected to the Ground (GND) pin.

- The Ground connection is made using Physical Pin 6 of the Raspberry Pi.

Telegram Bot Creation Steps

Step 1: Access BotFather (Search and Initialize)

- Open the Telegram application.

- Search for @BotFather.

- Open the BotFather chat.

- Click Start or type /start to view the available commands.

Step 2: Generate a New Bot (Bot Creation)

- Send the command /newbot.

- BotFather will ask you to enter a bot name.

- Enter MU_TYIT_LED_Bot or any desired name.

- Next, enter a unique username ending with bot.

Example: tyit_gpio_control_bot

Step 3: Copy HTTP API Token (API Security)

- After successful creation, BotFather generates a unique HTTP API Token.

- Copy the generated token.

- This token is required in the Python program to connect the Telegram Bot with Raspberry Pi.

import RPi.GPIO as GPIO

import telepot

import time

LED_PIN = 17

GPIO.setmode(GPIO.BCM)

GPIO.setwarnings(False)

GPIO.setup(LED_PIN, GPIO.OUT)

def handle(msg):

chat_id = msg['chat']['id']

command = msg['text'].lower()

if command == '/on':

GPIO.output(LED_PIN, GPIO.HIGH)

bot.sendMessage(chat_id, "LED ON")

elif command == '/off':

GPIO.output(LED_PIN, GPIO.LOW)

bot.sendMessage(chat_id, "LED OFF")

elif command == '/start':

bot.sendMessage(chat_id, "Send /on or /off")

TOKEN = 'YOUR_BOT_TOKEN'

bot = telepot.Bot(TOKEN)

bot.message_loop(handle)

print("Bot is running...")

try:

while True:

time.sleep(10)

except KeyboardInterrupt:

print("Bot stopped")

finally:

GPIO.cleanup()

Practical 3 Interfacing TM1637 with Raspberry Pi and Display current time.

Aim

To interface a TM1637 4-Digit 7-Segment LED Display Module with the GPIO pins of a Raspberry Pi 4 Model B, format the local Indian Standard Time (IST – Mumbai timezone) into 12-hour format with blinking colon indicators, and display continuous real-time updates.

Hardware Requirements

- Raspberry Pi 4 Model B

- TM1637 4-Digit 7-Segment Display Module (with Clock Colon)

- Female-to-Female Jumper Wires

- 5V 3A Power Supply

Software Requirements

- Raspberry Pi OS

- Python 3

- TM1637 Driver Library (pip install tm1637)

Physical Pin Connection Table

| TM1637 Module Pin | Raspberry Pi Physical Pin | Function |
| --- | --- | --- |
| VCC | Pin 1 | 3.3V Power |
| GND | Pin 6 | Ground (GND) |
| CLK (Clock) | Pin 40 | GPIO 21 (BCM 21) |
| DIO (Data I/O) | Pin 38 | GPIO 20 (BCM 20) |

Precautions

- Power Rating: Connect VCC to Physical Pin 1 (3.3V) to prevent sending 5V logic signals directly into Raspberry Pi GPIO pins.

- Polarity Check: Double-check VCC and GND connections before powering on to avoid damaging the display driver.

- Safe Wiring: Power down the Raspberry Pi before connecting jumper wires.

import time

import tm1637

tm = tm1637.TM1637(clk=21, dio=20)

print("Displaying Mumbai 12-Hour Time...")

try:

while True:

t = time.localtime()

hour = t.tm_hour % 12 or 12

tm.numbers(hour, t.tm_min, colon=True)

time.sleep(0.5)

tm.numbers(hour, t.tm_min, colon=False)

time.sleep(0.5)

except KeyboardInterrupt:

tm.write([0, 0, 0, 0])

print("Program stopped.")

o/p:
Displaying Mumbai 12-Hour Time...

Current System Time: 06:12 PM

TM1637 Hardware State:

[0] [6] : [1] [2]

(Colon blinking every 0.5 seconds)

Practical 4 Interface ADS1115 ADC with Raspberry Pi to Act as a Simple Digital Oscilloscope.

Aim

To interface the ADS1115 ADC module with a Raspberry Pi 4 Model B using the I2C protocol, generate a test PWM signal from a Raspberry Pi GPIO pin into the ADS1115 analog input pin (A0), sample the voltage waveform, and plot the real-time square wave using Python (Matplotlib) to function as a digital oscilloscope.

Hardware Requirements

- Raspberry Pi 4 Model B

- ADS1115 16-Bit 4-Channel ADC Module

- Female-to-Female Jumper Wires (4 wires for I2C/power + 1 signal wire)

- 5V 3A USB-C Power Supply

Software Requirements

- Raspberry Pi OS

- Python 3

- Adafruit CircuitPython ADS1x15 Library (pip install adafruit-circuitpython-ads1x15)

- Matplotlib Library (pip install matplotlib)

- RPi.GPIO Library

Physical Pin Connection Table

| ADS1115 Module Pin | Raspberry Pi Connection | Function | Physical Pin Number |
| --- | --- | --- | --- |
| VDD | 3.3V | Power Supply | Pin 1 |
| GND | GND | Common Ground | Pin 6 |
| SCL | GPIO 3 | I2C Clock Line | Pin 5 |
| SDA | GPIO 2 | I2C Data Line | Pin 3 |
| A0 | GPIO 18 | PWM Signal Input | Pin 12 |

import time

import board, busio

import RPi.GPIO as GPIO

import adafruit_ads1x15.ads1115 as ADS

from adafruit_ads1x15.analog_in import AnalogIn

import matplotlib.pyplot as plt

GPIO.setmode(GPIO.BCM)

GPIO.setup(18, GPIO.OUT)

pwm = GPIO.PWM(18, 2)

pwm.start(50)

i2c = busio.I2C(board.SCL, board.SDA)

ads = ADS.ADS1115(i2c)

channel = AnalogIn(ads, ADS.P0)

times, voltages = [], []

start = time.time()

try:

while time.time() - start < 3:

times.append(time.time() - start)

voltages.append(channel.voltage)

time.sleep(0.01)

finally:

pwm.stop()

GPIO.cleanup()

print("Captured samples. Generating plot...")

plt.plot(times, voltages)

plt.xlabel("Time (Seconds)")

plt.ylabel("Voltage (V)")

plt.title("Digital Oscilloscope")

plt.grid()

plt.show()

o/p:

ADS1115 Oscilloscope Sampling Started...

Captured samples. Generating plot...

Practical 5 Interface 16x2 LCD Display Directly with Raspberry Pi GPIO Pins to Display Text

Aim

To interface a 16×2 Character LCD display module with the GPIO pins of a Raspberry Pi 4 Model B using 4-bit parallel data mode and write a Python script using RPi.GPIO to display static and updated text messages.

Hardware Requirements

- Raspberry Pi 4 Model B

- 16×2 Character LCD Module (HD44780 compliant)

- 10 kΩ Potentiometer (for LCD contrast adjustment)

- Breadboard

- Male-to-Female / Male-to-Male Jumper Wires

- 5V 3A Power Supply

Software Requirements

- Raspberry Pi OS

- Python 3

- RPi.GPIO Library

Physical Pin Connection Table

| LCD Pin | Pin Name | Raspberry Pi Connection | Physical Pin |
| --- | --- | --- | --- |
| Pin 1 | VSS | Ground (GND) | 6 |
| Pin 2 | VDD | +5V Power | 2 |
| Pin 3 | V0 | Ground (GND) | 9 |
| Pin 4 | RS | GPIO 25 (BCM 25) | 22 |
| Pin 5 | RW | Ground (GND) | 6 |
| Pin 6 | E | GPIO 24 (BCM 24) | 18 |
| Pins 7–10 | D0–D3 | Unconnected | — |
| Pin 11 | D4 | GPIO 23 (BCM 23) | 16 |
| Pin 12 | D5 | GPIO 17 (BCM 17) | 11 |
| Pin 13 | D6 | GPIO 18 (BCM 18) | 12 |
| Pin 14 | D7 | GPIO 22 (BCM 22) | 15 |
| Pin 15 | BLA | +5V Power | 2 |
| Pin 16 | BLK | Ground (GND) | 14 |

import RPi.GPIO as GPIO

import time

RS, E = 25, 24

D4, D5, D6, D7 = 23, 17, 18, 22

GPIO.setmode(GPIO.BCM)

GPIO.setwarnings(False)

for pin in [RS, E, D4, D5, D6, D7]:

GPIO.setup(pin, GPIO.OUT)

def toggle():

GPIO.output(E, True)

time.sleep(0.0005)

GPIO.output(E, False)

time.sleep(0.0005)

def lcd_byte(data, mode):

GPIO.output(RS, mode)

for shift in (4, 0):

for pin, bit in zip([D4,D5,D6,D7], range(shift, shift+4)):

GPIO.output(pin, (data >> bit) & 1)

toggle()

def lcd_string(msg, line):

lcd_byte(line, False)

for ch in msg.ljust(16)[:16]:

lcd_byte(ord(ch), True)

try:

for cmd in [0x33, 0x32, 0x06, 0x0C, 0x28, 0x01]:

lcd_byte(cmd, False)

print("LCD Display Started...")

while True:

lcd_string("MU TYIT IOT LAB", 0x80)

lcd_string("DIRECT GPIO OK", 0xC0)

time.sleep(3)

lcd_string("RASPBERRY PI 4B", 0x80)

lcd_string("PRACTICAL 05 OK", 0xC0)

time.sleep(3)

except KeyboardInterrupt:

lcd_byte(0x01, False)

print("Program stopped.")

finally:

GPIO.cleanup()

o/p:

Direct GPIO 16x2 LCD Active...

0 – 3 Seconds:

MU TYIT IOT LAB

DIRECT GPIO OK

3 – 6 Seconds:

RASPBERRY PI 4B

PRACTICAL 05 OK

Program stopped.

GPIO Cleanup Complete.

Practical 6 Interfacing RC522 RFID Module with Raspberry Pi for Read and Write Operations

Practical 6: Interfacing RC522 RFID Module with Raspberry Pi

Aim

To interface the RC522 RFID module with Raspberry Pi using SPI communication and perform read and write operations on an RFID card.

Hardware Requirements

- Raspberry Pi 4 Model B

- RC522 RFID Module

- Mifare 1K RFID Card/Tag

- 7 Jumper Wires

- Power Supply

Software Requirements

- Raspberry Pi OS

- Python 3

- RPi.GPIO

- spidev

- mfrc522

Physical Pin Connection Table

| RC522 Pin | Raspberry Pi GPIO | Physical Pin |
| --- | --- | --- |
| SDA | GPIO 8 | 24 |
| SCK | GPIO 11 | 23 |
| MOSI | GPIO 10 | 19 |
| MISO | GPIO 9 | 21 |
| IRQ | Not Connected | — |
| GND | GND | 6 |
| RST | GPIO 25 | 22 |
| 3.3V | 3.3V | 1 |

Steps to Enable SPI

- Open Terminal.

- Enter sudo raspi-config.

- Select Interface Options → SPI.

- Select Yes to enable SPI.

- Reboot using sudo reboot.

import RPi.GPIO as GPIO

from mfrc522 import SimpleMFRC522

reader = SimpleMFRC522()

try:

while True:

print("\n1. Read Tag")

print("2. Write Tag")

print("3. Exit")

ch = input("Enter choice: ")

if ch == '1':

id, text = reader.read()

print("Tag UID:", id)

print("Data:", text.strip())

elif ch == '2':

data = input("Enter data: ")

reader.write(data)

print("Data written successfully!")

elif ch == '3':

print("Program stopped.")

break

else:

print("Invalid choice!")

finally:

GPIO.cleanup()

o/p:

1. Read Tag

2. Write Tag

3. Exit

Enter choice: 2

Enter data: Employee_ID_90112

Data written successfully!

Enter choice: 1

Tag UID: 874310562914

Data: Employee_ID_90112

Enter choice: 3

Program stopped.

Practical 7 Interfacing Fingerprint Sensor with Raspberry Pi

Aim

To interface an optical fingerprint sensor (R307/R305) with Raspberry Pi using a USB-to-TTL converter and perform fingerprint enrollment and matching using Python.

Hardware Requirements

- Raspberry Pi

- Optical Fingerprint Sensor (R307/R305)

- USB-to-TTL Converter

- 4 Jumper Wires

Software Requirements

- Raspberry Pi OS

- Python 3

- PySerial Library

- Adafruit CircuitPython Fingerprint Library

Connection Table

| Sensor Wire | Function | USB-to-TTL Pin |
| --- | --- | --- |
| Red | Power (5V) | 5V |
| Black | Ground | GND |
| Yellow/Green (TXD) | Sensor Transmit | RXD |
| White/Blue (RXD) | Sensor Receive | TXD |

Steps to Check USB Connection

- Connect the USB-to-TTL converter to the Raspberry Pi USB port.

- Open Terminal.

- Enter the command:

ls /dev/ttyUSB*

4. Usually, the output is /dev/ttyUSB0.

Menu Options

- Enroll Fingerprint

- Search/Match Fingerprint

- Exit

import sys

import serial

import time

import adafruit_fingerprint

uart = serial.Serial("/dev/ttyUSB0", 57600, timeout=1)

finger = adafruit_fingerprint.Adafruit_Fingerprint(uart)

def enroll():

id = int(input("Enter Fingerprint ID: "))

print("Place finger...")

while finger.get_image() != 0:

pass

finger.image_2_tz(1)

print("Remove finger...")

time.sleep(1)

while finger.get_image() != 2:

pass

print("Place same finger again...")

while finger.get_image() != 0:

pass

finger.image_2_tz(2)

if finger.create_model() == 0:

finger.store_model(id)

print("Fingerprint Enrolled!")

else:

print("Enrollment Failed!")

def search():

print("Place finger...")

while finger.get_image() != 0:

pass

finger.image_2_tz(1)

if finger.finger_search() == 0:

print("Match Found! ID:", finger.finger_id)

else:

print("No Match Found!")

try:

while True:

print("\n1. Enroll Fingerprint")

print("2. Search Fingerprint")

print("3. Exit")

ch = input("Enter choice: ")

if ch == '1':

enroll()

elif ch == '2':

search()

elif ch == '3':

print("Program stopped.")

break

else:

print("Invalid choice!")

except KeyboardInterrupt:

print("Program stopped.")

finally:

uart.close()

o/p:

1. Enroll Fingerprint

2. Search Fingerprint

3. Exit

Enter choice: 1

Enter Fingerprint ID: 5

Place finger...

Remove finger...

Place same finger again...

Fingerprint Enrolled!

Enter choice: 2

Place finger...

Match Found! ID: 5

Enter choice: 3

Program stopped.

Practical 8 Interfacing Raspberry Pi Camera Module to Capture Images and Video Recordings
Aim

To interface the Raspberry Pi Camera Module and use Picamera2 to capture photos, record videos, and exit safely.

Hardware Requirements

- Raspberry Pi

- Raspberry Pi Camera Module

- Power Supply

Software Requirements

- Raspberry Pi OS (Bookworm/Bullseye)

- Python 3

- Picamera2 Library

Physical Connection

- Power off the Raspberry Pi.

- Open the CSI camera port clip.

- Insert the ribbon cable with silver contacts facing the HDMI port and blue backing facing the USB/Ethernet ports.

- Lock the clip and power on the Raspberry Pi.

Software Setup

- Open Terminal.

- Run sudo raspi-config.

- Navigate to Interface Options → Camera.

- Enable the camera interface and reboot.

import sys

import time

from picamera2 import Picamera2

def setup_camera():

try:

cam = Picamera2()

cam.configure(cam.create_preview_configuration(

main={"size": (1280, 720)}))

cam.start()

return cam

except Exception as e:

print("Camera error:", e)

sys.exit(1)

def capture_image(cam):

name = f"photo_{int(time.time())}.jpg"

time.sleep(2)

cam.capture_file(name)

print("Image saved:", name)

def record_video(cam):

try:

duration = int(input("Enter duration: "))

if duration <= 0:

print("Invalid duration")

return

name = f"video_{int(time.time())}.h264"

cam.start_recording(name)

time.sleep(duration)

cam.stop_recording()

print("Video saved:", name)

except ValueError:

print("Invalid duration")

def main():

cam = setup_camera()

try:

while True:

print("\n1. Capture Image\n2. Record Video\n3. Exit")

choice = input("Enter choice: ")

if choice == "1":

capture_image(cam)

elif choice == "2":

record_video(cam)

elif choice == "3":

break

else:

print("Invalid choice")

finally:

cam.stop()

print("Camera closed")

main()

o/p:

1. Capture Image

2. Record Video

3. Exit

Enter choice: 1

Image saved: photo_1774812300.jpg

Enter choice: 2

Enter duration: 5

Video saved: video_1774812315.h264

Enter choice: 3

Camera closed

Practical 9 Interface Fingerprint Sensor and 16x2 LCD Display with Raspberry Pi to display Matched or Unmatched

Aim

To interface a fingerprint sensor and 16x2 LCD with Raspberry Pi and display fingerprint match status and User ID on the LCD.

Hardware Requirements

- Raspberry Pi

- R307/R305 Fingerprint Sensor

- USB-to-TTL Converter

- 16x2 LCD Module

- Jumper Wires

Software Requirements

- Raspberry Pi OS with Python 3

- Libraries: adafruit-circuitpython-fingerprint, pyserial, RPLCD

Physical Connections

Fingerprint Sensor to USB-to-TTL

| Sensor Pin | Converter Pin |
| --- | --- |
| VCC (Red) | 5V |
| GND (Black) | GND |
| TXD | RXD |
| RXD | TXD |

16x2 LCD to Raspberry Pi

| LCD Pin | Raspberry Pi |
| --- | --- |
| VSS (1) | GND (Pin 6) |
| VDD (2) | 5V (Pin 2) |
| V0 (3) | GND (Pin 9) |
| RS (4) | GPIO 22 (Pin 15) |
| RW (5) | GND (Pin 14) |
| E (6) | GPIO 18 (Pin 12) |
| D0–D3 (7–10) | Not Connected |
| D4 (11) | GPIO 23 (Pin 16) |
| D5 (12) | GPIO 24 (Pin 18) |
| D6 (13) | GPIO 25 (Pin 22) |
| D7 (14) | GPIO 27 (Pin 13) |
| A (15) | 5V (Pin 4) |
| K (16) | GND (Pin 20) |

import time

import serial

import RPi.GPIO as GPIO

import adafruit_fingerprint

from RPLCD.gpio import CharLCD

lcd = CharLCD(pin_rs=22, pin_e=18,

pins_data=[23, 24, 25, 27],

numbering_mode=GPIO.BCM, cols=16, rows=2)

uart = serial.Serial("/dev/ttyUSB0", 57600, timeout=1)

finger = adafruit_fingerprint.Adafruit_Fingerprint(uart)

def standby():

lcd.clear()

lcd.write_string("Biometric Access")

lcd.cursor_pos = (1, 0)

lcd.write_string("Place Finger...")

def show_result(match, uid=0):

lcd.clear()

if match:

lcd.write_string("STATUS: MATCHED")

lcd.cursor_pos = (1, 0)

lcd.write_string(f"User ID: #{uid}")

else:

lcd.write_string("STATUS:UNMATCHED")

lcd.cursor_pos = (1, 0)

lcd.write_string("Access Denied!")

time.sleep(3)

standby()

def main():

standby()

try:

while True:

if finger.get_image() == adafruit_fingerprint.OK:

if finger.image_2_tz(1) == adafruit_fingerprint.OK:

if finger.finger_search() == adafruit_fingerprint.OK:

show_result(True, finger.finger_id)

else:

show_result(False)

time.sleep(0.2)

except KeyboardInterrupt:

print("Exiting...")

finally:

lcd.clear()

lcd.write_string("System Off")

time.sleep(1)

lcd.clear()

uart.close()

GPIO.cleanup()

main()

o/p:
Biometric Access

Place Finger...

STATUS: MATCHED

User ID: #1

STATUS:UNMATCHED

Access Denied!


Practical 10 Create a motion detector by interfacing camera and 16x2 display with Raspberry Pi. Print the appropriate message on display.

Aim

To create a motion detection system using Raspberry Pi Camera and a 16x2 LCD. OpenCV detects motion and displays the status on the LCD.

Hardware Requirements

- Raspberry Pi

- Raspberry Pi Camera Module

- 16x2 LCD Display

- Jumper Wires

Software Requirements

- Raspberry Pi OS with Python 3

- Libraries: Picamera2, opencv-python, numpy, RPLCD

Camera Connection

- Power off the Raspberry Pi.

- Open the CSI camera port clip.

- Insert the ribbon cable with silver contacts facing the HDMI port and blue backing towards USB/Ethernet ports.

- Lock the clip and power on.

Camera Software Setup

- Open Terminal and run sudo raspi-config.

- Go to Interface Options → Camera.

- Enable the camera and reboot.

LCD Connection (4-bit Mode)

| LCD Pin | Raspberry Pi Connection |
| --- | --- |
| VSS (1) | GND (Pin 6) |
| VDD (2) | 5V (Pin 2) |
| V0 (3) | GND (Pin 9) |
| RS (4) | GPIO 22 (Pin 15) |
| RW (5) | GND (Pin 14) |
| E (6) | GPIO 18 (Pin 12) |
| D0–D3 (7–10) | Not Connected |
| D4 (11) | GPIO 23 (Pin 16) |
| D5 (12) | GPIO 24 (Pin 18) |
| D6 (13) | GPIO 25 (Pin 22) |
| D7 (14) | GPIO 27 (Pin 13) |
| A (15) | 5V (Pin 4) |
| K (16) | GND (Pin 20) |

import time

import cv2

import RPi.GPIO as GPIO

from picamera2 import Picamera2

from RPLCD.gpio import CharLCD

lcd = CharLCD(pin_rs=22, pin_e=18,

pins_data=[23, 24, 25, 27],

numbering_mode=GPIO.BCM, cols=16, rows=2)

def display(msg1, msg2=""):

lcd.clear()

lcd.write_string(msg1[:16])

lcd.cursor_pos = (1, 0)

lcd.write_string(msg2[:16])

def main():

cam = Picamera2()

cam.configure(cam.create_preview_configuration(

main={"size": (640, 480), "format": "RGB888"}))

cam.start()

time.sleep(2)

display("Calibrating...", "Keep Scene Still")

base = cv2.cvtColor(cam.capture_array(), cv2.COLOR_RGB2GRAY)

base = cv2.GaussianBlur(base, (21, 21), 0)

display("SYSTEM ARMED", "Monitoring...")

try:

while True:

frame = cv2.cvtColor(cam.capture_array(), cv2.COLOR_RGB2GRAY)

frame = cv2.GaussianBlur(frame, (21, 21), 0)

diff = cv2.absdiff(base, frame)

thresh = cv2.threshold(diff, 25, 255, cv2.THRESH_BINARY)[1]

thresh = cv2.dilate(thresh, None, iterations=2)

contours, _ = cv2.findContours(

thresh, cv2.RETR_EXTERNAL, cv2.CHAIN_APPROX_SIMPLE)

motion = any(cv2.contourArea(c) > 1000 for c in contours)

if motion:

display("MOTION DETECTED!", "Alert: Intruder")

else:

display("SYSTEM CLEAR", "Monitoring...")

base = cv2.addWeighted(base, 0.95, frame, 0.05, 0)

time.sleep(0.1)

except KeyboardInterrupt:

print("Exiting...")

finally:

cam.stop()

lcd.clear()

lcd.write_string("System Shutdown")

GPIO.cleanup()

main()

o/p:

SYSTEM ARMED

Monitoring...

MOTION DETECTED!

Alert: Intruder

SYSTEM CLEAR

Monitoring...